# AMRITA SCHOOL OF ENGINEERING

23RAI398 MINI PROJECT

# ASSIGNMENT – 2

# Structured & Algorithmic Robotic Concept Development

## 1. Assignment Objective

The objective of this assignment is to develop and evaluate multiple robotic design concepts for the proposed duck-inspired companion robot using structured engineering methods.

This includes defining functional architecture, algorithmic behavior, and alternative design approaches, followed by a systematic evaluation to select the most feasible concept for prototype development.

## 2. Project Details

Project Title: QUANC

Team Members:

- A C Anirudh			| BL.EN.U4RAE23003

- Nihal P			| BL.EN.U4RAE23029

- S Venkata Rami Reddy	| BL.EN.U4RAE23038

## 3. Problem Re-Definition

### 3.1 Refined Problem Statement

Develop a compact indoor robotic companion (QUANC) that can detect environmental hazards and basic human presence while providing simple, friendly interaction for elderly individuals living alone, without relying on intrusive surveillance or medical monitoring systems.

### 3.2 Target Users / Application Environment

Target Users: Elderly individuals living independently and their caregivers.

Application Environment: Indoor home environments such as bedrooms or living rooms.

Operating Conditions: Normal household conditions with moderate lighting, typical indoor temperatures, and limited obstacles.

### 3.3 Constraints

Budget: Limited student prototype budget (~₹30k–₹40k).

Size: Compact indoor robot (< 40 cm height).

Power: Battery-powered system with limited runtime.

Environment: Designed only for indoor home environments.

Safety: Must operate safely around humans and furniture.

## 4. Functional Decomposition of Your Assigned Robot

### 4.1 Overall System Function

Provide friendly companionship and environmental awareness for an elderly individual living alone by sensing surroundings, detecting presence, and communicating important events.

### 4.2 Functional Breakdown

| Main Function | Sub Functions | Expected Output |
| --- | --- | --- |
| Sensing | Gas detection, temperature & humidity sensing, light sensing, human presence detection | Environmental data and presence status |
| Decision / Control | Process sensor data, detect abnormal conditions, decide robot behavior | Control commands and alert decisions |
| Actuation | Biped movement, head movement, eye expression display | Physical movement and expressive feedback |
| Communication | Send alerts to caregiver, provide status indication | Notifications and system feedback |
| Power Management | Battery supply, voltage regulation, power monitoring | Stable power distribution to subsystems |

### 4.3 Functional Block Diagram

## 5. Algorithmic Design

### 5.1 Control Flowchart

### 5.2 Pseudocode

Start system

Initialize sensors and actuators

While system is running:

Read environmental sensors

Read presence detection sensors

If environmental hazard detected:

Send alert to caregiver

If human presence detected:

perform interaction behavior

(eye movement / head motion)

Else:

remain idle or perform slow movement

Check system health

If fault detected:

stop movement

enter safe state

End loop

## 6. Concept Generation

| Parameter | Concept 1 – Mechanical | Concept 2 – Sensor/AI | Concept 3 – Hybrid (Selected) |
| --- | --- | --- | --- |
| Working Principle | Mechanically driven system with predefined behavior | Intelligence driven by sensing and perception | Balanced system combining sensing, control, and mobility |
| Locomotion Type | Wheeled | Minimal / static | Biped (duck-inspired) |
| Sensor Strategy | Basic sensors (gas, temperature, proximity) | Camera, microphone, environmental sensors | Environmental + basic CV + proximity |
| AI / Processing | Simple rule-based | CV + rule-based processing | Rule-based + state machine |
| Actuation Method | DC motors + servo | Limited actuation (mostly expressive) | Biped legs + head/eye actuation |
| Autonomy Level | Low to semi-autonomous | High | Semi-autonomous |
| Advantages | Low cost, easy to build, reliable | High intelligence, better interaction | Balanced performance, good interaction, research value |
| Limitations | Less interactive, low intelligence | High cost, complex, compute-heavy | Integration complexity, walking stability issues |

## 7. Concept Sketches (Mandatory)

Battery 450mah

Actuator

Sensor placement

Periscope Camera

10.1-inch HD Touchscreen

Dual Qualcomm Processors

Actuator

Communication Module

Face Joint

Control Unit

Sensory Unit

Power Unit

Camera(s)

Neck Joint

Ankle Joint

Knee Joint

Hip Pitch Joint

Hip Yaw Joint

## 8. Concept Evaluation & Selection

### 8.1 Evaluation Criteria

| Criteria | Weight (%) |
| --- | --- |
| Technical Feasibility | 20 |
| Cost | 15 |
| Ease of Fabrication | 15 |
| Autonomy Capability | 15 |
| Safety | 10 |
| Scalability | 10 |
| User Acceptance | 15 |

### 8.2 Decision Matrix

| Criteria | Weight | Concept 1 | Concept 2 | Concept 3 |
| --- | --- | --- | --- | --- |
| Technical Feasibility | 20 | 9 | 6 | 7 |
| Cost | 15 | 9 | 5 | 7 |
| Ease of Fabrication | 15 | 9 | 6 | 7 |
| Autonomy Capability | 15 | 5 | 9 | 7 |
| Safety | 10 | 8 | 7 | 7 |
| Scalability | 10 | 6 | 8 | 8 |
| User Acceptance | 15 | 5 | 7 | 9 |
| Total Score | 100 | 7.4 | 6.8 | 7.5 |

### 8.3 Selected Concept

| Criteria | Weight | Concept 1 | Concept 2 | Concept 3 |
| --- | --- | --- | --- | --- |
| Technical Feasibility | 20 | 9 | 6 | 7 |
| Cost | 15 | 9 | 5 | 7 |
| Ease of Fabrication | 15 | 9 | 6 | 7 |
| Autonomy Capability | 15 | 5 | 9 | 7 |
| Safety | 10 | 8 | 7 | 7 |
| Scalability | 10 | 6 | 8 | 8 |
| User Acceptance | 15 | 5 | 7 | 9 |
| Total Score | 100 | 7.4 | 6.8 | 7.5 |

## 9. Proposed System Architecture (Selected Concept Only)

### 9.1 Hardware Architecture

## Controller: ESP32 /Pi Pico 2S W + Raspberry Pi (for CV)

## Sensors:

## Gas sensor (MQ series)

## Temperature & Humidity (DHT11/DHT22)

## Ambient light sensor

## Camera (for basic CV)

## Proximity / ToF sensor

## IMU (for balance)

## Actuators:

## Servo motors (legs, head)

## DC motors (if hybrid motion used)

## Motor Drivers: DRV8833 / similar

## Communication Module: Wi-Fi (ESP32) / Bluetooth

## Power System: Li-ion battery + voltage regulators

### 9.2 Software Architecture

- Control Logic: Finite State Machine (FSM)

- Autonomy Level: Semi-autonomous

- Algorithm Type: Rule-based with basic CV integration

- Data Flow:
Sensors → Processing → Decision (FSM) → Actuation → Feedback

- Modules:

- Sensor data acquisition

- Environment analysis (threshold-based)

- Presence detection (CV)

- Behavior control (state machine)

- Communication (alerts to caregiver)

## 10. Risk & Feasibility Analysis

| Risk | Probability | Impact | Mitigation Strategy |
| --- | --- | --- | --- |
| Biped instability (falling) | High | High | Start with constrained gait, low speed, wide stance; test on flat surfaces |
| Power consumption too high | Medium | Medium | Use efficient servos, optimize duty cycles, sleep modes |
| Sensor noise / false alerts | Medium | Medium | Filtering, threshold tuning, multi-sensor validation |
| CV not reliable in low light | Medium | Low | Use fallback (proximity/IR), limit CV dependency |
| Integration complexity | High | High | Modular development (test sensing, control, locomotion separately) |
| Weight vs actuator torque mismatch | Medium | High | Keep lightweight design, calculate torque before selection |
| Budget overrun | Medium | Medium | Prioritize core components, avoid overengineering |

Feasibility Summary:
The system is feasible as a prototype if scope is controlled—focus on basic walking, sensing, and rule-based behavior, while keeping CV and advanced features minimal.

## 11. Innovation Statement

The proposed QUANC robot introduces a non-invasive approach to elderly assistance by combining companionship with environmental awareness in a single system. Unlike traditional assistive robots that focus on medical monitoring or surveillance, this design emphasizes user comfort, emotional acceptance, and simplicity.

The key innovation lies in the duck-inspired bipedal architecture, which provides a pet-like presence while enabling controlled mobility within indoor spaces. The system integrates lightweight sensing, rule-based decision making, and expressive interaction, avoiding dependence on complex AI while still delivering meaningful functionality.

Additionally, the robot is designed as a context-aware observer rather than an active controller, ensuring that it supports users without interfering with their daily routines. This balance between mobility, sensing, and interaction makes the system both technically achievable and socially acceptable.

Overall, the project contributes a practical and human-centered robotic design suitable for real-world deployment at low cost.

## 12. Sustainability & Ethical Considerations

- Energy Efficiency:
Designed for low power consumption using efficient sensors and controlled actuator usage to extend battery life.

- Environmental Impact:
Uses off-the-shelf, reusable components; compact design reduces material usage.

- Safety:
Operates at low speed with safe failure behavior (stop and alert); suitable for indoor human environments.

- Privacy & Ethics:
Avoids continuous surveillance and data recording; processes minimal data and only sends event-based alerts, ensuring user privacy and dignity.

- Social Impact:
Promotes independence of elderly individuals while providing reassurance to caregivers without replacing human interaction.