# AMRITA SCHOOL OF ENGINEERING

23RAI398 MINI PROJECT

# ASSIGNMENT – 3

# Sensors, Actuators & Control Architecture (Conceptual)

A C Anirudh		| BL.EN.U4RAE23003

Nihal P			| BL.EN.U4RAE23029

S Venkata Rami Reddy	| BL.EN.U4RAE23038

# 1. System Overview:

| Aspect | Description |
| --- | --- |
| System Description | QUANC is a duck-inspired indoor companion robot that performs environmental monitoring, presence detection, and simple interaction using rule-based control. |
| Key Functionality | Monitor environment (gas, temperature, light), detect human presence, provide expressive interaction, send alerts to caregivers. |
| Input Parameters | Sensor data from gas, temperature, humidity, light, camera/proximity, touch/IMU. |
| Output Actions | Eye/head movement, robot locomotion, alerts/notifications to caregiver. |
| Operating Environment | Indoor home environment; moderate lighting, flat surfaces, human presence. |
| Constraints | Limited budget, compact size, battery power, indoor-only operation. |
| Assumptions | Stable indoor conditions, minimal obstacles, basic user interaction, simplified sensing models. |

# 2. Sensor Selection:

| Sensor Name | Type (Analog/Digital) | Purpose | Working Principle | Range/Accuracy | Interface | Justification |
| --- | --- | --- | --- | --- | --- | --- |
| MQ-2 Gas Sensor | Analog | Detect gas leakage | Resistance change with gas concentration | ~200–10000 ppm | ADC | Low cost, reliable for hazard detection |
| DHT11 / DHT22 | Digital | Temperature & humidity sensing | Capacitive + thermistor | ±2°C, ±5% RH | Single-wire | Simple and sufficient for indoor monitoring |
| LDR (Light Sensor) | Analog | Ambient light detection | Resistance varies with light | Low precision | ADC | Cheap and effective for light sensing |
| Camera Module | Digital | Presence / posture detection | Image capture + CV processing | Algorithm dependent | CSI / USB | Enables visual awareness |
| Ultrasonic / ToF Sensor | Digital | Distance / proximity sensing | Time-of-flight measurement | 2–400 cm | GPIO / I2C | Works as fallback for presence detection |
| IMU (MPU6050) | Digital | Orientation & balance | Accelerometer + gyroscope | High sensitivity | I2C | Essential for biped stability |
| Touch Sensor | Digital | User interaction | Capacitive sensing | Binary | GPIO | Enables simple interaction |
| Microphone Module | Analog/Digital | Sound/event detection | Converts sound waves to electrical signals | Depends on module | ADC / GPIO | Detects calls, sudden sounds, or activity |

# 3. Actuator Selection:

| Actuator Name | Type | Function | Specifications | Power Requirement | Control Method | Justification |
| --- | --- | --- | --- | --- | --- | --- |
| BLDC / DC Motor + Gearbox | Rotary | Hip pitch (forward/back leg motion) | High torque, geared output | High | PWM + Motor Driver | Provides required torque and efficiency for primary locomotion |
| Servo Motor (High Torque) | Rotary | Hip roll (side balance) | ~10–20 kg·cm torque | Medium | PWM | Precise lateral balance control |
| Pneumatic Spring | Linear (Passive) | Knee joint support | Force based on body weight | None (passive) | Mechanical | Reduces control complexity and energy usage |
| Servo Motor (Medium Torque) | Rotary | Ankle stabilization | ~5–10 kg·cm torque | Medium | PWM | Maintains balance and foot orientation |
| Servo Motor (SG90) | Rotary | Head and eye movement | Low torque | Low | PWM | Enables expressive interaction |
| Buzzer / Speaker | Audio | Feedback / alerts | Basic audio output | Low | GPIO / PWM | Provides sound-based feedback |
| OLED / LED Display | Visual | Eye expressions / status | Small display module | Low | I2C / SPI | Improves user interaction and feedback |

# 4. Control Unit:

| Parameter | Details |
| --- | --- |
| Controller Name | ESP32 + Raspberry Pi 5 |
| Type | Microcontroller (ESP32) + Microprocessor (Raspberry Pi) |
| Processing Capability | ESP32: Dual-core, ~240 MHz; Pi: Multi-core, GHz-level |
| Input/Output Interfaces | GPIO, ADC, PWM, I2C, SPI, UART |
| Communication Protocols | I2C, SPI, UART, WiFi, Bluetooth |
| Power Consumption | ESP32: Low–Medium; Pi: Medium–High |
| Cost | ESP32: ₹500; Pi: ₹10,000 |

## Justification Table

| Criteria | Explanation |
| --- | --- |
| Why this controller? | ESP32 handles real-time control; Raspberry Pi handles CV and higher-level processing |
| Compatibility | Supports all selected sensors and actuators with standard interfaces |
| Scalability | Easy to expand with additional sensors or modules |
| Cost-effectiveness | ESP32 keeps base cost low; Pi is optional based on need |
| Ease of Implementation | Strong community support and available libraries |

# 5. System Block Diagram:

# 6. Control Logic: Flowchart or pseudocode

Start

INITIALIZE sensors

INITIALIZE actuators

WHILE system_is_running:

// --- Read Sensors ---

sensor_data = READ_SENSORS()

camera      = sensor_data.camera

gas         = sensor_data.gas

temperature = sensor_data.temperature

light       = sensor_data.light

proximity   = sensor_data.proximity

// --- Check Environment ---

IF NOT ENVIRONMENT_IS_NORMAL(sensor_data) THEN

SEND_ALERT_TO_CAREGIVER()

CONTINUE   // skip rest of loop, go to next cycle

ENDIF

// --- Human Detection ---

IF HUMAN_PRESENT(camera, proximity) THEN

EXECUTE_INTERACTION_BEHAVIOR()

// e.g., eye motion, head movement

ELSE

EXECUTE_IDLE_OR_NAVIGATION()

ENDIF

// --- Fault Detection ---

IF FAULT_DETECTED() THEN

ENTER_SAFE_MODE()

STOP_MOVEMENT()

ENDIF

// --- Loop Delay / Monitoring ---

WAIT(dt)

END WHILE

END

# 7. Integration:

Sensor Interaction
Sensors continuously collect environmental and interaction data:

- Analog signals: Gas sensor, LDR, microphone

- Digital signals: IMU, camera, ToF, touch, temperature/humidity

These inputs represent environmental state, user presence, and robot condition.

Controller Processing

The ESP32 processes real-time sensor data using rule-based logic and a finite state machine (FSM).

- Environmental data is compared against thresholds

- Presence data triggers interaction behaviors

- CV (if used) runs on Raspberry Pi and sends processed results to ESP32

Decision-making is based on IF–ELSE conditions and system states.

Actuator Response

Based on decisions:

- Hip (BLDC/DC + gearbox): drives forward motion

- Hip roll & ankle (servo): maintain balance

- Knee (passive pneumatic spring): supports motion

- Head/eyes: provide expressive feedback

- Buzzer/display: alert or communicate status

Outputs result in movement, interaction, or alerts to caregiver.

8. Safety & Feasibility:
Safety

- Operates at low speed to prevent injury or damage

- Implements fail-safe behavior: stop motion on fault or instability

- Uses lightweight structure to reduce impact risk

- Electrical components are properly regulated and insulated

- Avoids continuous recording → privacy-safe design

Feasibility

- Technically feasible as a prototype with simplified walking and rule-based control

- Biped design is challenging but manageable with slow gait and passive knee support

- Uses readily available components (ESP32, sensors, servos)

- Cost can be controlled by limiting advanced features (CV optional)

- Modular design allows incremental development and testing