# AMRITA SCHOOL OF ENGINEERING

23RAI398 MINI PROJECT

# ASSIGNMENT – 4

# Embodiment Design & Material Selection for Robotic System

A C Anirudh		| BL.EN.U4RAE23003

Nihal P			| BL.EN.U4RAE23029

S Venkata Rami Reddy	| BL.EN.U4RAE23038

# 1. System Layout & Configuration

| Aspect | Description |
| --- | --- |
| Overall System Layout | Compact duck-inspired biped robot with sensors mounted on the head/torso, controller in central body, battery at lower center for stability |
| Type of Robot | Bipedal companion robot |
| Degrees of Freedom (DOF) | ~4–6 DOF (hip pitch, hip roll, ankle, head movement) |
| Mobility Mechanism | Biped walking using hip-driven motion with passive knee support |
| Structural Configuration | Box-like torso with two articulated legs and a head module |
| Weight Considerations | Approx. 2–3 kg for stability and actuator feasibility |

# 2. Embodiment Design

| Component | Function | Design Description | Sketch/CAD Reference |
| --- | --- | --- | --- |
| Chassis / Torso | Support all components | Box-type central body housing controller, sensors, and wiring | Fig. 1 |
| Leg Assembly | Enable locomotion | Two legs with hip (motor + servo), passive knee (spring), and servo-driven ankle | Fig. 2 |
| Head Module | Interaction & sensing | Mounted on top with camera, microphone, and expressive display | Fig. 3 |
| Sensor Mounts | Hold sensors | Front-facing mounts for gas, light, and proximity sensors | Fig. 4 |
| Battery Housing | Power supply | Located at lower torso for center of gravity stability | Fig. 5 |

# 3. Functional Mapping

| Function | Component Used | Description |
| --- | --- | --- |
| Motion | BLDC/DC Motor + Gearbox, Servo Motors, Pneumatic Knee | Enables biped walking, balance, and posture control |
| Sensing | Gas sensor, DHT sensor, LDR, Camera, IMU, Microphone, ToF | Collects environmental and presence data |
| Control | ESP32 + Raspberry Pi | Processes sensor inputs and controls system behavior |
| Interaction | Servo (head), Display, Buzzer | Provides visual and audio feedback |
| Support | Chassis / Frame | Holds and integrates all components |

# 4. Material Selection (Basic Table Example)

| Component | Candidate Materials | Selected Material | Properties Considered | Justification |
| --- | --- | --- | --- | --- |
| Chassis / Structural Frame | Aluminum, Steel | Aluminum | Strength-to-weight, rigidity | Provides strong lightweight support for entire robot |
| Leg Structure | Aluminum, Steel | Aluminum | Load-bearing capacity, weight | Handles dynamic loads during walking |
| Outer Housing (Body + Head) | PLA, ABS, PETG | 3D Printed (PLA / PETG) | Lightweight, manufacturability | Easy prototyping, customizable design |
| Sensor Mounts | PLA, ABS | 3D Printed (PLA) | Ease of fabrication | Quick iteration and integration |
| Feet | TPU, Silicone, Rubber | TPU / Silicone | High friction, compliance | Improves grip and absorbs impact during walking |
| Battery Enclosure | ABS, PLA | 3D Printed (PLA/ABS) | Insulation, durability | Integrated within housing for compact design |

# 5. Material Selection Using Ashby Charts

### Step 1: Define Function, Objective, Constraints

| Parameter | Description |
| --- | --- |
| Function | Support structural load while enabling lightweight biped movement |
| Objective | Minimize weight while maintaining sufficient strength and stiffness |
| Constraints | Strength ≥ required load capacity, low cost, easy manufacturability |

### Step 2: Material Selection Table

| Material | Density (ρ) | Young’s Modulus (E) | Strength (σ) | Cost | Performance Index (E/ρ) | Rank |
| --- | --- | --- | --- | --- | --- | --- |
| Aluminum Alloy | Medium | High | Medium | Medium | High | 1 |
| Mild Steel | High | High | High | Low | Medium | 2 |
| PLA / ABS | Low | Low | Low | Low | Low | 3 |

Aluminum is selected for structural components due to its high stiffness-to-weight ratio, while 3D printed plastics are used for non-load-bearing parts to reduce cost and improve manufacturability.

# 6. Preliminary Design Calculations

| Parameter | Value | Assumption |
| --- | --- | --- |
| Load | ~2.5 kg | Total robot weight |
| Torque Required (Hip) | ~1.5–2 Nm | Based on leg length and load distribution |
| Servo Torque (Ankle/Hip Roll) | ~8–12 kg·cm | For balance and stabilization |
| Speed | Slow gait (~10–20 steps/min) | Stability prioritized over speed |
| Factor of Safety | 2 | Standard for prototype design |

# 7. Sustainability Considerations

| Aspect | Description |
| --- | --- |
| Material Recyclability | Aluminum structure is recyclable; 3D printed parts can be reprinted or reused |
| Energy Efficiency | Uses low-power sensors and controlled actuator usage to reduce consumption |
| Environmental Impact | Battery-operated system with minimal emissions |
| Manufacturing | 3D printing reduces material waste and enables rapid prototyping |

# 8. Design Constraints & Assumptions

| Constraint Type | Description |
| --- | --- |
| Mechanical | Limited load capacity and stability challenges in biped design |
| Economic | Budget constrained to student-level prototype (~₹10k–₹20k) |
| Environmental | Designed for indoor operation only |
| Manufacturing | Limited access to advanced fabrication (CNC, precision machining) |

| Assumption | Description |
| --- | --- |
| Operating Conditions | Flat indoor surfaces with minimal obstacles |
| User Behavior | Minimal direct interaction; passive operation |
| Component Availability | Standard components available locally/online |
| Simplifications | Walking gait, sensing, and control kept simple for feasibility |